#### Approved

With the decision of the Management Board of AS Eesti Raudtee

of 5 November 2019 No. 501/3.2

#### Railway network statement

(period from 8 December 2019 to 12 December 2020)

Prepared by:	Eesti Raudtee

Telliskivi 60/2, 15073, TALLINN

Telephone: 615 8660

E-mail: raudtee@evr.ee

# Table of Contents:

# General information	4

- Technical specifications of the railway infrastructure and conditions for access to it	4

## Volume of the capacity to be allocated	4

## General description of the capacity to be allocated	4

  - Technical specification of the railway network	6

## Connection to railway infrastructure owned by third parties, exceptions in using the railway infrastructure for rail transport	9

## Technological restrictions preventing the use of capacity	11

## Railway vehicle requirements	12

2. Principles for the determination of railway infrastructure user fees	17

2.1	Calculation of railway infrastructure costs	17

2.2	User fees for basic services of the railway infrastructure and mark-ups of user fees	18

2.3	User fees for extra and support services and service facilities of the railway infrastructure	18

2.4	Calculation of fixed assets and capital expenditure	18

2.5	Calculation of reasonable operating profit	19

2.6	Unit cost of the railway infrastructure user fees	19

2.7	Reduction of user fees and mark-ups	19

2.8	Determination of user fees	19

2.9	Reservation fee	20

3. Principles and criteria for the allocation of capacity	20

3.1	Principles of the capacity allocation procedure	20

3.2	Applicants for capacity and criteria for satisfying the applications	21

## 3.3	Deadlines of the capacity allocation procedure	23

## 3.4	Preparation of timetable	23

## 3.5	Coordination process for the allocation of capacity	24

3.6	Declaring the capacity depleted	26

3.7	Allocation of capacity in the event that capacity has been declared depleted	26

3.8	Application for and allocation of single partial railway capacities intended for specific purposes	27

3.9	Contract for use of railway infrastructure	28

3.10	Obligation to use capacity	30

3.11	Unallocated capacity	31

3.12	Plan for increasing capacity	31

4. Application for an operating licence and a safety certificate	32

4.1	Operating licence	32

4.2	Safety certificate	33

5. Procedure of dispute resolution and filing of complaints regarding access to the railway infrastructure and services and the performance scheme	33

6. Access to service facilities	34

6.1	Service facilities	34

6.2	Operator of service facilities	37

6.3	Access to service facilities and the services provided therein	37

7. Allocation procedure of international train paths	37

# 7.1	Passenger train paths	37

7.2	Freight train paths	38

# 8. Market segments	38

# 9. Annexes to the railway network statement	38

# General information

The railway network statement was prepared by AS Eesti Raudtee (hereinafter Eesti Raudtee) based on consultations with the Consumer Protection and Technical Regulatory Authority and other known interested persons. The railway network statement of 2019/2020 was approved by the 30 October 2018 decision No. 449/9 and updated by the 5 November 2019 decision No. 501/3.2 of the Management Board of Eesti Raudtee.

The railway network statement reflects the conditions of access to the railway infrastructure of Eesti Raudtee in the timetabling period from 08 December 2019 to 12 December 2020.

The railway network statement has been prepared in conformity with the Railways Act and legal acts issued on the basis thereof, considering, among other things, the requirements and aims of Directives (EU) 2008/57, (EU) 2012/34, and (EU) 2016/798.

Terms and definitions used in the railway network statement originate from legal acts, rules and guidelines.

The railway network statement is available at an address approved by the Competition Authority, the website of Eesti Raudtee http://www.evr.ee/et/arikliendile#liiklusgraafikud. The paper version of the railway network statement is available for 6.4 euros from Eesti Raudtee at Telliskivi 60/2, Tallinn.

If necessary, additional information and explanations about the railway network statement will be provided to railway undertakings by Eesti Raudtee: Telliskivi 60/2, 15073 Tallinn, phone: 615 8660.

Eesti Raudtee keeps the railway network statement up to date and has the right to make amendments to it, if required. Amendments made to the railway network statement are approved by Eesti Raudtee. A notice of amending the railway network statement will be published on the Eesti Raudtee website at http://www.evr.ee/et/arikliendile#liiklusgraafikud and communicated to all persons who have taken out a paper version of the railway network statement by the time of amendment.

- Technical specifications of the railway infrastructure and conditions for access to it

## Volume of the capacity to be allocated

    - The volume of the capacity to be allocated and the data on the basis of which it is calculated have been provided in Annex 2 to the railway network statement.

    - Pursuant to Annex 2 of the railway network statement, the volume of capacity to be allocated is the largest possible total number of trains which may pass through a railway section between the station of origin and the station of destination in 24 hours (from 17.00 to 17.00), and the basis for the calculation of which are the criteria provided in subsection 51 (2) of the Railways Act.

    - The speed limits established by Eesti Raudtee (available on the website http://www.evr.ee/et/arikliendile#eeskirjad-ja-tasud) are the basis for the calculation of the average travelling time.

## General description of the capacity to be allocated

    - The basis for the organisation of railway traffic on the Eesti Raudtee railway infrastructure is the timetable that links the operation of all IM/RUs operating in the field of rail maintenance and rail transport. The timetable is prepared, coordinated and approved pursuant to the Railways Act.

    - Management of the train traffic of Eesti Raudtee occurs through train dispatchers. Organisation of the train traffic of Eesti Raudtee is divided into four dispatcher areas:                               1. dispatcher area – Tallinn-Tapa, 2. dispatcher area – Tapa-Narva, 3. dispatcher area – Tapa-Tartu-Koidula (Pechory-Pskovskiye), 4. dispatcher area – Tartu-Valga-Koidula (Pechory-Pskovskiye), Tallinn-Paldiski-Turba.

    - Eesti Raudtee railway infrastructure utilises the following train traffic signalling equipment:

- automatic lockout in the Tallinn – Keila, Tallinn – Tapa, Tapa – Narva, Tapa – Tartu, Tartu – Palupera, Valga – Koidula, Lagedi – Muuga, Ülemiste – Maardu and Valga – Lugaži (166,300 km to the border) areas;

- semi-automatic lockout in the Tartu – Reola, Koidula – Pechory-Pskovskiye (605,026 km to the border), Narva – Ivangorod-Narvski (314,934 km to the border), Palupera – Valga, Keila – Riisipere, Keila – Paldiski, Tallinn – Tallinn-Väike and Ülemiste – Liiva (5,536 km to the border) areas;

- combined road lockout in the Reola - Koidula area;

Among those areas with automatic lockout and combined road lockout, Tallinn – Tapa, Tapa – Tartu, Tartu – Palupera, Valga – Koidula,  Reola - Koidula and Tapa – Narva areas have been equipped with operations control systems.

1.2.4 The railway infrastructure of Eesti Raudtee only uses locomotive trains once in the following railway sections:

- Klooga - train stop Kloogaranna,

- Riisipere - train stop Turba.

1.2.5	Employees responsible for the safety of railway traffic are guided in their tasks by legal acts, rules for the technical use of railways, and the procedures, operating rules and its annexes established on the basis of these. Specific professional duties regarding the receiving and dispatching of trains and letting them pass through, and the movement of other railway vehicle units at the station (block post) have been established in the regulations concerning the technical organisation of the station (block post) and the job descriptions of employees.

1.2.6 In case of accidents which have occurred on the Eesti Raudtee railway infrastructure, Eesti Raudtee adheres to the agreements made with railway undertakings that are established in the contract for use of railway infrastructure regarding remedying the consequences and restoration of railway traffic.

1.2.7 Eesti Raudtee establishes the conditions for the maintenance of railway infrastructure, railway traffic management and granting of use of the railway infrastructure to other persons.  The operating rules established by Eesti Raudtee and its annexes are published on the Eesti Raudtee website http://www.evr.ee/et/arikliendile#eeskirjad-ja-tasud.

1.2.8 	The technical data of the Eesti Raudtee railway infrastructure have been provided in Annex 3 of the railway network statement.

1.2.9 Operations enabled on the Eesti Raudtee railway civil engineering works (passenger transport operations upon the embarkation and disembarkation of passengers on the train, freight transport operations and the admission and dispatching of trains, shunting, coupling and decoupling rolling stock) have been established in Annex 1 of the railway network statement.

1.2.10 Boundaries of paths with other public railroads have been provided in clause 2.3.6 of the operating rules of Eesti Raudtee.

1.2.11	Boundaries of railway infrastructure and railway traffic management between Eesti Raudtee and neighbouring states have been established with the relevant interstate border agreements. Interstate railway border crossing points are Narva – Ivangorod-Narvski and Koidula – Pechory-Pskovskiye with the Russian Federation and Valga – Lugaži with the Republic of Latvia.

  - Technical specification of the railway network

1.3.1 Technical data of the railway infrastructure:

- total length of railway paths,	800 km

of this, the total length of double track paths	94 km

total length of electrified paths	2225 km

- number of stations	61

- usable lengths of station tracks	850 m

- usable lengths of border station tracks:

Narva	1500 m

Koidula	1500 m

Valga	850 m

The railway infrastructure of Eesti Raudtee utilises track gauges of 1520 and 1524 mm. Tracks are divided as follows:

- main tracks (open tracks and tracks in stations together with crossovers of the main track),

- station tracks (besides main tracks, all sidetracks in the station area together with other tracks built for a specific purpose).

The main type of rail on main tracks is R50, R65 or UIC60. The real load of any axle of a railway vehicle cannot exceed 32 tonnes.

Eesti Raudtee establishes the speed limits on open tracks and station tracks for the timetabling period with a directive of the Director of Infrastructure (available on the Eesti Raudtee website http://www.evr.ee/et/arikliendile#eeskirjad-ja-tasud).

1.3.2 Radio communication (radio communication for trains and shunting):

The Eesti Raudtee railway infrastructure uses the digital radio system MotoTRBO (Motorola technology) for radio communication with trains and for shunting which utilises a frequency range of 136–174 MHz with a channel bandwidth of 12.5 kHz.

Equipment for radio communication with trains must operate in duplex mode and equipment for radio communication for shunting must operate in simplex mode. Before any kind of railway vehicle can be used on the Eesti Raudtee railway infrastructure, it must be equipped with a Motorola DMxxxx radio station that has a subscription (with a unique assigned ID-code) previously registered with Eesti Raudtee.

Use of other radio communications equipment may occur only under prior coordination with Eesti Raudtee. The setting parameters of radio communication equipment are released by Eesti Raudtee immediately before their use on the Eesti Raudtee railway infrastructure. The functioning of radio communication equipment in the Eesti Raudtee radio communication network must be inspected prior to its first-time use. It is forbidden to change the setting parameters without the consent of Eesti Raudtee. The use of radio communication and procedural rules are described in the operating rules of Eesti Raudtee at http://www.evr.ee/et/arikliendile#eeskirjad-ja-tasud.

Railway vehicle units from foreign states entering border stations (Narva, Koidula, Valga) must be equipped with radio stations using analogue technology that operate on the frequency of 2.130 MHz. Portable radio stations will be issued to foreign railway workers by their employer (clause 2.2.7 of the operating rules).

1.3.3 ALS coding systems

The Eesti Raudtee railway infrastructure is equipped with 50 Hz ALSN automatic locomotive signalling track fixtures. All locomotives and multiple-unit trains travelling on the Eesti Raudtee railway infrastructure must be equipped with an on-board system that can receive ALSN codes (ALSN, VEPS, KLUB-U or КПД-3ПА, KADRI)

The board system used for receiving the ALSN codes must have gone through a conformity assessment by the Consumer Protection and Technical Regulatory Authority before its initial implementation on Eesti Raudtee’s infrastructure. The scope of the conformity assessment includes checking, among other things, the conformity of the onboard equipment, its compliance with the railway vehicles and the infrastructure.

The cockpit of a self-moving special railway vehicle must include railway transport radio communications equipment, rolling stock driver’s vigilance control device with an auto-stop device and a speedometer or an on-board computer that register the parameters while moving. The requirements of this clause do not expand to a high rail vehicle, except for in regards to the railway transport radio equipment device requirement. The on-board computer or speedometer located in the cockpit of a self-moving

special railway vehicle must at least record the following parameters:

- speed of movement;

- direction of travel and distance;

- pressure in the braking system;

- vigilance control device for the driver of the rolling stock;

- date and time in case of an on-board computer;

- traffic light display in case of locomotive’s automatic signalisation equipment.

1.3.4 Inspection systems of railway vehicles:

The Eesti Raudtee railway infrastructure utilises a system for detecting the overheating of bushings and wheels of railway vehicles. The devices placed on the railway infrastructure after about every 30 km are able to communicate information to traffic management personnel and railway dispatcher.

| HOTBOX system | Odd track I / Even track II | Inspected technical km | Warning km/pkt for trains |
| --- | --- | --- | --- |
| # 1) Raasiku – Lagedi | # (odd track I) | # 125.314 | # 126 km 4 pkt |
| # 2) Lagedi – Raasiku | # (even track II) | # 125.317 | # 126 km 4 pkt |
| # 3) Raasiku – Kehra | # (even track II) | # 137.289 | # 138 km 3 pkt |
| # 4) Aegviidu – Kehra | # (odd track I) | # 148.489 | # 149 km 5 pkt |
| # 5) Tapa – Aegviidu | # (odd track I) | # 176.520 | # 177 km 6 pkt |
| # 6) Aegviidu – Tapa | # (even track II) | # 176.512 | # 177 km 6 pkt |
| # 7) Tapa – Nõmmküla |  | # 318.028 | # 319 km 1 pkt |
| # 8) Tapa – Kadrina |  | # 185.574 | # 186 km 6 pkt |
| # 9) Rakvere – Vaeküla |  | # 213.467 | # 214 km 5 pkt |
| # 10) Sonda – Kiviõli |  | # 239.648 | # 240 km 7 pkt |
| # 11) Kohtla – Jõhvi |  | # 262.102 | # 263 km 2 pkt |
| # 12) Vaivara – Auvere |  | # 294.246 | # 295 km 3 pkt |
| # 13) Kiltsi – Rakke |  | # 343.444 | # 344 km 5 pkt |
| 14) Pedja – Jõgeva |  | 376.739 | 377 km 8 pkt |
| 15) Tabivere – Kärkna |  | 409.441 | 410 km 5 pkt |
| 16) Tartu – Kärkna |  | 421.800 | 422 km 9 pkt |
| 17) Tartu – Reola |  | 5.501 | 6 km 6 pkt |
| 18) Tartu – Ropka |  | 431.301 | 432 km 4 pkt |
| 19) Elva – Palupera |  | 456.509 | 457 km 6 pkt |
| 20) Puka – Keeni |  | 479.111 | 480 km 2 pkt |
| 21) Vastse-Kuuste – Põlva |  | 30.981 | 31 km 10 pkt |
| 22) Veriora – Orava |  | 64.383 | 65 km 4 pkt |
| 23) Valga – Sangaste |  | 508.407 | 509 km 5 pkt |
| 24) Valga – Karula |  | 516.281 | 517 km 3 pkt |
| 25) Valga – Lugaži |  | 165.643 | 166 km 7 pkt |
| 26) Antsla – Sõmerpalu |  | 544.247 | 545 km 3 pkt |
| 27) Võru – Lepassaare |  | 575.909 | 576 km 10 pkt |
| 28) Valingu - Keila |  | 79.310 | 80 km 4 pkt |
| 29) Keila - Klooga |  | 2.002 | 3 km 1 pkt |

1.3.5 Power supply

1.3.5.1 The operating voltage of the Eesti Raudtee power supply is 0.4/6/10/35/110 kV. Tallinn – Keila – Kloogaranna – Paldiski – Riisipere – Turba and Tallinn – Aegviidu railway sections have been electrified. The plants for transforming and carrying electric power for train haulage are supplied with power by 3 kV traction substations (Järve, Keila and Raasiku) that are connected to the 35 kV(20) power network of Elektrilevi OÜ and the 110 kV power network of Elering AS.

1.3.5.2 Power to railway consumers and communication and protection equipment is supplied via the 10 kV, 6 kV and 0.4 kV distribution network of Eesti Raudtee. The security of electricity supply of traffic-related railway consumers is guaranteed with backup feeders. Power to some protection and communication equipment is ensured with generators, uninterruptible power supplies or a combination of these.

On electrified paths, the minimum permitted period of time between electric trains in one way is: on the Keila-Järve section – 7 minutes, on the Järve-Tallinn section – 8 minutes, on the Tallinn-Raasiku section – 8 minutes, on the Raasiku-Aegviidu section – 16 minutes. Using two electric trains in a coupled composition is permitted with the same interval on the condition that the coupled composition must alternate with a regular composition. Using two electric trains in a coupled composition successively is permitted with an interval twice as long.

On the Paldiski-Keila and Riisipere-Keila sections, simultaneous acceleration is permitted for up to three electric trains at a time; at the same time, two trains may be on the same railway section.

When dispatching trains from Keila station to Riisipere and Paldiski stations, an interval of at least 10 minutes between electric trains must be guaranteed.

An electric railway vehicle must correspond to the prescribed working parameters of the power supply system. The electric railway vehicle power supply system guarantees the operation of trains with a power of at least 2 MW without the need for an amperage limit upon adhering to the required interval between electric trains. The conditions will be specified in the relevant contract for use of railway infrastructure, if necessary.

Upon using the Aegviidu parallel post on the Raasiku-Aegviidu section, the following conditions apply:

- upon departure from Aegviidu station, the power consumed during acceleration cannot exceed 80% of the maximum power of the electric train;

- upon the acceleration of an electric train near the Aegviidu station on the odd track of the Raasiku-Aegviidu section, a second electric train is allowed to simultaneously accelerate when departing from Kehra station or Parila train stop.

The following conditions apply on the Ülemiste-Raasiku section:

- upon using two electric trains in a coupled composition while departing from Lagedi station, the power consumed during acceleration cannot exceed 80% of the maximum power of the electric train.

Upon using regenerative braking of electric trains as service brakes, it is not possible to recuperate the released energy back into the primary power supply network. Upon electric braking, it is permitted to direct the released energy to other trains in the same region.

## Connection to railway infrastructure owned by third parties, exceptions in using the railway infrastructure for rail transport

    - International railway transport

      - The boundaries between the railway infrastructures of Eesti Raudtee and other railway infrastructure managers and the conditions for connection between railways are determined with contracts and/or legal instruments establishing boundaries.

      - The organisation of railway traffic between Eesti Raudtee and other railway infrastructure managers and the use of signalisation and communication devices in railway traffic are regulated by legal acts and contracts entered into between Eesti Raudtee and other railway infrastructure managers or possessors of railway infrastructure.

      - Boundaries of railway infrastructure and organisation of train traffic between Eesti Raudtee and foreign railway infrastructure managers are established with international agreements and legal acts, including the 25 June 2002 regulation No. 205 of the Government of Estonia, “Border crossing points open for international traffic”.

      - Train traffic with the Republic of Latvia is governed by the rail transport organisation agreement between AS Latvijas Dzelzcelš and Eesti Raudtee entered into on 2 December 2008 and the railway transport cooperation agreement between the governments of the Republic of Estonia, the Republic of Latvia and the Republic of Lithuania adopted on 7 December 2011 which entered into force on 1 September 2013. The aforementioned agreements have been provided in the annex to the operating rules of Eesti Raudtee. The transfer of the obligation to transport goods and liability for carriages is performed in Valga station, which is situated in the territory of the Republic of Estonia.

      - Train traffic with Russia is governed by the 25 February 1992 agreement on the direct railway connection between the Ministry of Roads of the Russian Federation and the Ministry of Transport and the Interior of the Republic of Estonia, and the temporary railway border agreement attached to it (available on the website http://www.evr.ee/et/arikliendile#eeskirjad-ja-tasud– clause 58 of the Annex to the operating rules). The transfer of the obligation to transport goods and liability for carriages is performed in Pechory station in the Tartu - Pechory and Valga - Pechory directions, which is situated in the territory of the Russian Federation, and in Narva station in the Narva - Ivangorod direction, which is situated in the territory of the Republic of Estonia. In addition to the aforementioned, rail traffic with Russia is more specifically governed by the cooperation agreement for the organisation of rail transport entered into by Открытое акционерное общество “Российские железные дороги” (OAO RŽD) and Eesti Raudtee on 9 February 2010.

      - Operations in Pechory and Narva border stations related to the direct connection of the railway between Estonia and Russia are performed in the simultaneous presence of the relevant structural units of Eesti Raudtee and OAO RŽD (the relevant authorisation originates from the temporary border agreement specified in clause 1.4.1.5). The use of (types of) locomotives, maximum speeds, weights and lengths of trains are coordinated pursuant to the border agreement between the parties in every preparation of the timetable.

      - The international passenger and goods transport on the Estonian territory is organised pursuant to international agreements.

      - If a railway undertaking wishes to use the railway infrastructure for providing international transport services, they shall:

        - Be guided, in border stations, by contract No. 380 of 22 May 2009 and contract No. 77 of 9 February 2010 entered into by Eesti Raudtee and OAO RŽD, and the agreement “On the organisation of rail transport” entered into by Eesti Raudtee and AS Latvijas Dzelzcelš on 2 December 2008, to ensure conformity with all relevant requirements and conditions in force in the Republic of Estonia, the Russian Federation and the Republic of Latvia regarding providing a border service in the border stations in the territory of the Republic of Estonia and the Russian Federation (Narva, Koidula, Pechory), including requirements originating from international agreements and agreements between railway administrations of the Republic of Estonia, the Russian Federation and the Republic of Latvia; performing the activities related to the relevant international rail transport operations (including freight and passenger transport), which are the technical and commercial receiving and dispatching of trains (including technical and commercial inspection, removal of deficiencies, finalising documents, data processing, etc.) and paying for the provided services. Eesti Raudtee organises the provision of a locomotive service between the Koidula and Pechory border stations based on the locomotive service contract entered into with a railway undertaking as a result of a public procurement procedure. Eesti Raudtee organises the provision of border services by persons with whom there are contracts for the provision of the relevant services as a result of a public procurement procedure. Service providers have been indicated in Annex 6 to the railway network statement;

        - have staff who are trained (have received official training) and, if necessary, are duly accredited by Eesti Raudtee, as the railway administration of Estonia, to timely perform all required activities around the clock related to the organisation of train and goods transport operations in border and destination stations pursuant to the agreements with and requirements of the Council for Railway Transport;

        - ensure the interoperability of their information system and data exchange with the relevant systems of Eesti Raudtee and pursuant to the procedure of application of the interoperability of railway systems and special agreements, and enter into a contract for using the traffic organisation information system with Eesti Raudtee;

        - ensure the timely and seamless organisation of work of trains and carriages, and entry of activities and data to the Eesti Raudtee Customer information system that is connected to the information systems of other states, to collect statistical and other data on trains and freight carriages pursuant to the decisions of the Council for Railway Transport.

1.4.2 Domestic railway transport

1.4.2.1 The important conditions for the rolling stock moving from non-public railway to the railway infrastructure of Eesti Raudtee and vice versa shall be included in the agreement concluded between Eesti Raudtee and non-public railway regarding the connections between railway infrastructures as well as shunting. This agreement must be concluded in writing.

1.4.2.2 Use capacity does not need to be applied for separately:

- for rescue train or auxiliary locomotive transport;

- locomotive or engine train transport in the case of an accident or for the replacement of a locomotive driving a train or an engine train with a technical fault, so goods or passenger transport could be continued;

- work train (rolling stock used for railway maintenance works) transport to and from the work object for the purpose of managing the railway infrastructure of Eesti Raudtee.

# Technological restrictions preventing the use of capacity

    - The technological limitations reducing volume of capacity or preventing the use of capacity are primarily speed limits established on a part of capacity, the time spent on the acceleration and braking of trains, technological possessions, deviations in train traffic, etc.

    - Eesti Raudtee has the right to perform large-scale railway repair works that restrict railway traffic for up to 24 hours. The schedule of works is specified by railway sections and it is coordinated with the Consumer Protection and Technical Regulatory Authority and IM/RUs providing rail transport in that section.

    - Eesti Raudtee may temporarily shut down rail traffic on the railway infrastructure for 24 hours or limit it significantly, if this is necessary:

      - for the elimination of imminent danger to people, property or the environment caused due to the technical state of railway infrastructure or railway vehicles;

      - for the removal of an extraordinary traffic obstruction caused by an event, a traffic accident or natural disaster, sudden change in weather conditions or other circumstances;

      - in other justified and unavoidable cases.

    - Railway traffic may be substantially restricted or temporarily closed for more than one twenty-four hour period pursuant to the conditions of the 18 March 2004 regulation No. 75 of the Government of Estonia, “The procedure for substantial restriction and temporary closure of rail traffic”. Eesti Raudtee notifies railway undertakings of a substantial restriction or temporary closure of railway traffic in writing, indicating the reason of the substantial restriction or temporary closure, its beginning and end dates, and traffic management during the substantial restriction or temporary closure of railway traffic.

    - In planning their work, railway undertakings must consider the railway maintenance works preventing the use of railway infrastructure during the railway maintenance work season, and changes in the timetable arising therefrom.

    - For the period of railway maintenance works planned in Annex 3 to the railway network statement, Eesti Raudtee prepares alternative timetables for train traffic pursuant to timetabling guidelines and capacity of railway sections, where is prescribed a possibility of shutting down train traffic for the duration of works, letting trains pass through more frequently in the period preceding the window, timely bringing and removal of work trains before and after the window, and, after the window has concluded, letting trains pass through with the speed corresponding to the technical state of the tracks.

    - Short-term traffic interruptions caused by short-term railway maintenance works and the train traffic procedure stemming therefrom is planned in the course of daily planning of the train operations timetable depending on the capacity in use on the railway section.

  - Railway vehicle requirements

1.6.1	Railway vehicles used on the Eesti Rolling stock railway infrastructure, locomotive crews and special vehicle operators must meet the requirements provided in legal acts, operating rules of Eesti Raudtee and other rules established by Eesti Raudtee. Upon loading goods to be carried on railway vehicles, a railway undertaking is required to follow, in addition to the abovementioned requirements, international requirements and rules (above all, requirements established in the SMGS, the European Union and the Council for Railway Transport).

    - The used railway vehicle must conform to the requirements of the technical rules for use of the railways, including:

      - being designed to be used on tracks with a gauge of 1520/1524 mm;

      - being equipped with type SA-3 automatic couplers; in coordination with the Consumer Protection and Technical Regulatory Authority, it is permitted to use other types of automatic couplers. To couple a vehicle like this to a vehicle equipped with the main-type SA-3 automatic couplers, it must be equipped with assistive fittings;

      - having automatic brakes;

      - having external dimensions that do not exceed the 1-T size limits;

      - means of transport must be equipped with stationary radio communication that operates on the frequency used on the railway infrastructure and the radio stations used on the means of transport must have ID-numbers issued by Eesti Raudtee that allow for entry into the radio communications system;

      - the real load of any axle of a railway vehicle cannot exceed 32 tonnes.

    - Using a series of locomotive equipped differently to what is provided in clause 1.6 of the railway network statement is only permitted with the consent of Eesti Raudtee. The relevant application must be attached to the capacity allocation application. Eesti Raudtee has the right to refuse an authorisation for use of a locomotive or other means of transport indicated in the application if that series is incompatible with the Eesti Raudtee railway infrastructure.

    - The following norms for the weight and length of freight trains are primarily permitted on the Eesti Raudtee railway infrastructure:

| Area | Locomotive | Weight of train in tonnes |  |  |  | Train length in conventional wagons* |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | STANDARD |  | MAXIMUM |  |  |  |
|  |  | odd | even | odd | even | odd | even |
| Narva – Tapa | C 36-7i | 5000 | 4200 | 5500 | 4200 | 57 | 57 |
|  | C 30 M | 3000 | 3000 | 3000 | 3000 | 57 | 57 |
|  | 2TE116 | 4700 | 4600 | 5500 | 4600 | 57 | 57 |
|  | TEM-2U/2UM/18 | 4200 | 4200 | 4650 | 4650 | 57 | 57 |
|  | TEM7 | 3200 | 3200 | 3900 | 3200 | 57 | 57 |
|  | TEM TMH | 2200 | 2200 | 2200 | 2200 | 57 | 57 |
|  | ČME 3 | 1500 | 1500 | 2000 | 2000 | 57 | 57 |
|  | M62 | 2200 | 2200 | 2200 | 2200 | 57 | 57 |
|  | DF7G-E | 3000 | 3000 | 3000 | 3000 | 57 | 57 |
| Narva – Vaivara | C 36-7i | 6000 | 5100 | 6000 | 5100 | 57 | 57 |
|  | C 30 M | 5000 | 4000 | 5600 | 4000 | 57 | 57 |
|  | 2TE116 | 6000 | 5100 | 6000 | 5100 | 57 | 57 |
|  | TEM-2U/2UM/18 | 2300 | 1600 | 2300 | 1600 | 57 | 57 |
|  | TEM7 | 3200 | 3200 | 5600 | 3200 | 57 | 57 |
|  | TEM TMH | 4300 | 4000 | 5400 | 4000 | 57 | 57 |
|  | ČME 3 | 1500 | 1500 | 2000 | 2000 | 57 | 57 |
|  | M62 | 2200 | 2200 | 2200 | 2200 | 57 | 57 |
|  | DF7G-E | 5000 | 4000 | 5600 | 4000 | 57 | 57 |
| Tapa – Ülemiste | C 36-7i | 5500 | 5000 | 6000 | 5000 | 57 | 57 |
|  | C 30 M | 5500 | 3800 | 5500 | 3800 | 57 | 57 |
|  | 2TE116 | 4700 | 4600 | 6000 | 5500 | 57 | 57 |
|  | TEM TMH | 2200 | 2200 | 2200 | 2200 | 57 | 57 |
|  | ČME 3 | 2000 | 1500 | 2500 | 2000 | 57 | 57 |
|  | M62 | 2300 | 2000 | 2300 | 2000 | 57 | 57 |
|  | DF7G-E | 5500 | 4000 | 6000 | 4500 | 57 | 57 |
| Ülemiste – Maardu | C 36-7i | 3700 | 4600 | 3700 | 4600 | 57 | 57 |
|  | C 30 M | 2500 | 2500 | 2500 | 2500 | 57 | 57 |
|  | 2TE116 | 4600 | 5000 | 4600 | 5000 | 57 | 57 |
|  | TEM-2U/2UM/18 | 1300 | 1500 | 1300 | 1500 | 57 | 57 |
|  | TEM TMH | 2500 | 2200 | 2500 | 2200 | 57 | 57 |
|  | ČME 3 | 1900 | 2000 | 1900 | 2000 | 57 | 57 |
|  | M62 | 1900 | 2250 | 1900 | 2250 | 57 | 57 |
|  | DF7G-E | 2500 | 2500 | 2500 | 2500 | 57 | 57 |
| Lagedi – Muuga | C 36-7i | 5000 | 4200 | 5500 | 4200 | 57 | 57 |
|  | C 30 M | 3500 | 2500 | 3500 | 2500 | 57 | 57 |
|  | 2TE116 | 4700 | 4200 | 5500 | 4600 | 57 | 57 |
|  | TEM-2U/2UM/18 | 3800 | 2500 | 3800 | 2500 | 57 | 57 |
|  | TEM TMH | 2200 | 2200 | 2200 | 2200 | 57 | 57 |
|  | ČME 3 | 1500 | 1500 | 1500 | 1500 | 57 | 57 |
|  | M62 | 2250 | 1900 | 2250 | 1900 | 57 | 57 |
|  | DF7G-E | 3500 | 2500 | 3500 | 2500 | 57 | 57 |
| Ülemiste – Tallinn | C 36-7i | 6000 | 3800 | 6000 | 3800 | 57 | 57 |
|  | C 30 M | 5500 | 2800 | 5500 | 2800 | 57 | 57 |
|  | 2TE116 | 6000 | 5000 | 6000 | 5000 | 57 | 57 |
|  | TEM-2U/2UM/18 | 1500 | 1300 | 1500 | 1300 | 57 | 57 |
|  | TEM TMH | 2200 | 2200 | 2200 | 2200 | 57 | 57 |
|  | ČME 3 | 1500 | 1500 | 1700 | 1500 | 57 | 57 |
|  | M62 | 2300 | 1700 | 2300 | 1700 | 57 | 57 |
|  | DF7G-E | 5500 | 2800 | 5500 | 2800 | 57 | 57 |
| Tallinn – Paldiski | C 36-7i | 3800 | 3500 | 4000 | 4000 | 45 | 45 |
|  | C 30 M | 2800 | 2800 | 2800 | 2800 | 45 | 45 |
|  | 2TE116 | 4200 | 4200 | 4300 | 5000 | 45 | 45 |
|  | TEM-2U/2UM/18 | 1500 | 1600 | 1500 | 1600 | 45 | 45 |
|  | TEM7 | 3300 | 3300 | 3300 | 3300 | 45 | 45 |
|  | TEM TMH | 2200 | 2200 | 2200 | 2200 | 45 | 45 |
|  | TEM 7 | 3300 | 3300 | 3300 | 3300 | 45 | 45 |
|  | ČME 3 | 1500 | 1500 | 1700 | 1800 | 45 | 45 |
|  | M62 | 1500 | 1500 | 1500 | 1500 | 45 | 45 |
|  | DF7G-E | 2800 | 2800 | 2800 | 2800 | 45 | 45 |
| Keila – Riisipere | C 36-7i | 3800 | 3500 | 4000 | 4000 | 45 | 45 |
|  | C 30 M | 4000 | 4000 | 4000 | 4000 | 45 | 45 |
|  | TEM TMH | 2200 | 2200 | 2200 | 2200 | 45 | 45 |
|  | ČME 3 | 1500 | 1500 | 2000 | 2000 | 45 | 45 |
|  | M62 | 1500 | 1500 | 1500 | 1500 | 45 | 45 |
|  | DF7G-E | 4000 | 4000 | 4000 | 4000 | 45 | 45 |
| Tapa – Tartu | C 36-7i | 5000 | 4200 | 5500 | 5000 | 57 | 57 |
|  | 2TE116 | 4700 | 4700 | 5500 | 5000 | 57 | 57 |
|  | TEM-2U/2UM/18 | 2100 | 2100 | 2400 | 2400 | 57 | 57 |
|  | TEM TMH | 2200 | 2200 | 2500 | 2500 | 57 | 57 |
|  | ČME 3 | 1500 | 1200 | 2000 | 1700 | 57 | 57 |
|  | M62 | 2000 | 2000 | 2000 | 2000 | 57 | 57 |
|  | DF7G-E | 3000 | 3000 | 3000 | 3000 | 57 | 57 |
| Orava – Tartu | C 36-7i | 4500 | 4500 | 4500 | 5000 | 57 | 57 |
|  | 2TE116 | 5200 | 5200 | 5200 | 5600 | 57 | 57 |
|  | TEM-2U/2UM/18 | 2000 | 2000 | 2000 | 2000 | 57 | 57 |
|  | TEM TMH | 2200 | 2200 | 2200 | 2500 | 57 | 57 |
|  | ČME 3 | 1800 | 1800 | 2000 | 2000 | 57 | 57 |
|  | M62 | 1500 | 1800 | 1500 | 1800 | 57 | 57 |
|  | DF7G-E | 3000 | 3000 | 3000 | 3000 | 57 | 57 |
| Koidula – Orava | C 36-7i | 3800 | 4200 | 3800 | 4200 | 57 | 57 |
|  | 2TE116 | 5200 | 5200 | 5200 | 5600 | 57 | 57 |
|  | TEM-2U/2UM/18 | 2000 | 2000 | 2200 | 2000 | 57 | 57 |
|  | TEM TMH | 2200 | 2500 | 2200 | 2800 | 57 | 57 |
|  | ČME 3 | 1800 | 1800 | 2000 | 2000 | 57 | 57 |
|  | M62 | 1500 | 1500 | 1500 | 1500 | 57 | 57 |
|  | DF7G-E | 7500 | 6000 | 7500 | 6000 | 57 | 57 |
| Koidula – Valga | C 36-7i | 4200 | 4200 | 4200 | 4200 | 57 | 57 |
|  | 2TE116 | 5200 | 5200 | 5200 | 5900 | 57 | 57 |
|  | TEM-2U/2UM/18 | 2000 | 2000 | 2100 | 2100 | 57 | 57 |
|  | TEM TMH | 2200 | 2200 | 2200 | 2500 | 57 | 57 |
|  | ČME 3 | 1700 | 1700 | 1700 | 1700 | 57 | 57 |
|  | M62 | 1600 | 1600 | 1600 | 1600 | 57 | 57 |
|  | DF7G-E | 2800 | 3000 | 2800 | 3000 | 57 | 57 |
| Valga – Tartu | C 36-7i | 4200 | 4200 | 4200 | 4200 | 57 | 57 |
|  | 2TE116 | 4700 | 4700 | 5500 | 5000 | 57 | 57 |
|  | TEM-2U/2UM/18 | 2000 | 2100 | 2100 | 2200 | 57 | 57 |
|  | TEM TMH | 2200 | 2200 | 2500 | 2500 | 57 | 57 |
|  | ČME 3 | 1000 | 1200 | 1500 | 1700 | 57 | 57 |
|  | M62 | 1600 | 2000 | 1600 | 2000 | 57 | 57 |
|  | DF7G-E | 2650 | 2800 | 2650 | 2800 | 57 | 57 |
| Koidula – Pechory | C 36-7i | 6000 | 6000 | 6000 | 6000 | 57 | 57 |
|  | TEM TMH | 5300 | 5000 | 5300 | 5000 | 57 | 57 |
|  | 2TE116 | 7500 | 6000 | 7500 | 6000 | 57 | 57 |
|  | 2TEM-2U/2UM/18 | 7000 | 7000 | 7000 | 7000 | 57 | 57 |
|  | TEM7 | 6000 | 6200 | 6000 | 6200 | 57 | 57 |
| Tallinn – Tallin-Väike | ČME 3 | 1200 | 1200 | 1200 | 1200 | 48 | 48 |
|  | TEM 4 | 1000 | 1000 | 1000 | 1000 | 48 | 48 |

The gross weight of a train being hauled in the service area cannot exceed the maximum permitted gross weight assigned to the locomotive being used. The traffic of longer trains occurs in coordination with the traffic service of Eesti Raudtee.

1.6.5	Locomotives used for freight transport on the Eesti Raudtee railway infrastructure:

| Type of locomotive | Locomotive length in conventional wagons |
| --- | --- |
| C 36-7i | 1.46 |
| 2TE116 | 2.59 |
| TEM-2U/2UM/18 | 1.21 |
| ČME 3 | 1.23 |
| M62 | 1.24 |
| TEM TMH | 1.21 |
| DF7G-E | 1.39 |
| TEM 4 | 1.21 |
| TEM7 | 1.54 |
| C 30 M | 1.55 |

*The length of a standard unit is 14 m.

    - To move a railway vehicle required for an IM/RU or possessor of railway vehicle for rail transport on the Eesti Raudtee railway infrastructure, it must be driven by a railway vehicle driver who has a valid locomotive driver’s licence of Estonia or another European Union Member State, and a certificate issued by a railway undertaking that shows which section of infrastructure the driver is permitted to drive on and which railway vehicle it is licensed to drive. Drivers who have a certificate are not required to have a separate work permit from Eesti Raudtee. Drivers of special railway vehicles who have a professional certificate compliant with subsection 44 (4) of the Railways Act must pass a practical driving test in the relevant region and pass an examination on the regulations concerning the technical organisation in the relevant stations and local working conditions to get a work permit for working on the Eesti Raudtee railway infrastructure. Examination of knowledge occurs after every four years. Information about the expiration of work permits is available at: http://www.evr.ee/ärikliendile/eeskirjad-ja-tasud.

    - An examiner recognised by the Consumer Protection and Technical Regulatory Authority shall assess the required general professional knowledge and knowledge related to the railway vehicle and the Eesti Raudtee railway infrastructure to issue the certificate provided in clause 1.6.5. The examination board of Eesti Raudtee shall assess the knowledge related to the Eesti Raudtee railway infrastructure that is required to issue a work permit.

    - A shunter may perform the duties of the assistant locomotive driver upon shunting a locomotive at the station or upon departure to open track. A shunter may perform the duties of an assistant locomotive driver if they have the relevant profession.

    - In passenger transport, railway vehicle combinations complying with the following requirements may be used on the railway infrastructure of Eesti Raudtee:

1.6.9.1 Electric trains:

| Type of train | Number of carriages | Gross weight in tonnes | Train length in conventional wagons |
| --- | --- | --- | --- |
| Flirt EMU | 3 | 130 | 4.12 |
| Flirt EMU | 4 | 159 | 5.36 |

1.6.9.2 Diesel trains:

| Type of train | Number of carriages | Gross weight in tonnes | Standard length |
| --- | --- | --- | --- |
| Flirt DMU | 2 | 124 | 3.25 |
| Flirt DMU | 3 | 148 | 4.28 |
| Flirt DMU | 4 | 176 | 5.31 |
| DR1A | 6 | 312 | 10.68 |
| DR1A | 5 | 266 | 8.9 |
| DR1A | 4 | 220 | 7.12 |
| DR1A | 3 | 174 | 5.34 |
| DR1A | 2 | 128 | 3.56 |
| DR1B | 6 | 316 | 10.68 |
| DR1B | 5 | 270 | 8.9 |
| DR1B | 4 | 224 | 7.12 |
| DR1B | 3 | 178 | 5.34 |
| DR1B | 2 | 132 | 3.56 |
| DR1BJ | 3 | 158 | 5.34 |
| DR1BJ | 2 | 112 | 3.56 |
| DR1 | 4 | 232 | 7.12 |

1.6.9.3 Carriage compositions:

1.6.9.3.1 Locomotives:

| Type | Estimated weight in tonnes | Locomotive length in conventional wagons |
| --- | --- | --- |
| TEP 70 | 135 | 1.46 |

        - Passenger carriages, restaurant, baggage and other special-purpose passenger carriages that correspond to the requirements of the rules in force in Estonia (including the Rules for using passenger carriages in international transport (Правила пользования пассажирскими вагонами в международном сообщении (ПППВ)).

        - A passenger transport undertaking must coordinate the schematics and lengths of train compositions navigating on the Eesti Raudtee railway infrastructure.

1.6.10 The conditions for loading and securing goods, and for transporting goods with nonstandard measurements or that are heavy loads or are loaded on conveyors have been provided in clause 4.4 of the operating rules of Eesti Raudtee.

2.  Principles for the determination of railway infrastructure user fees

The user fees for basic services and extra services ensuring access and access support services, and mark-ups of basic services and extra services ensuring access and access support services of the Eesti Raudtee railway infrastructure, as well as user fees for single railway capacities intended for specific purposes and user fees of service facilities are determined on the basis of the 19 August 2016 regulation No. 51 of the Minister of Economic Affairs and Infrastructure, “Methodology for the calculation of user fees of railway infrastructure” (hereinafter methodology).

  - Calculation of railway infrastructure costs

    - Eesti Raudtee has developed principles based on which they differentiate between costs related to the provision of basic services, extra services and support services; these principles are updated as required pursuant to the best practical knowledge and international practices. Costs of the railway infrastructure manager are ascribed to the specific service that has caused these costs to occur. Every cost is either directly or proportionally ascribed to a single service.

    - Before determining the user fees and mark-ups of the railway infrastructure and user fees of service facilities, the Consumer Protection and Technical Regulatory Authority and the operator of service facilities assess the expenses of the railway infrastructure manager separately for every timetabling period based on the data of the financial year preceding the timetabling period and specify the expenditure figures that the user fees are based on with the price indices published by Statistics Estonia which determine the level of inflation for the next economic year.

    - The user fees and mark-ups for basic services and extra services ensuring access and access support services are determined based on the same principle across the whole railway network of the railway infrastructure manager, and the railway infrastructure user fee must ensure the use of public railroad without discrimination to all users of the railway infrastructure capacity.

    - The railway infrastructure user fees are paid to the railway infrastructure manager who finances their economic activities on account of this.

    - Service facility user fees are paid to the operator of the service facility who finances their economic activities on account of this. The operator of service facilities is Eesti Raudtee pursuant to subsection 58 (1(5)) of the Railways Act.

2.2 User fees for basic services of the railway infrastructure and mark-ups of user fees

2.2.1 The costs of allowing the use of basic services of the railway infrastructure consist of costs directly related to the organisation of railway traffic (direct costs) to which the Consumer Protection and Technical Regulatory Authority may establish mark-ups on the basis of efficient, transparent and non-discriminatory principles, taking account of the capability of persons operating in railway market segments to pay these. The user fees and mark-ups of user fees cannot exceed the total cost associated with the provision of basic services ensuring access (including capital expenditure and reasonable operating profit).

2.2.2 Pursuant to subsection 59 (3(1)) of the Railways Act, the Consumer Protection and Technical Regulatory Authority may establish a maximum rate of mark-ups to basic services ensuring access, taking account of the capability of persons operating in railway market segments to pay these, at the same time ensuring optimal competition in the railway market segments.

2.2.3	The direct costs of allowing the use of basic services of the railway infrastructure are calculated based on the European Commission Implementing Regulation (EU) 2015/909 which concerns methods for the calculation of direct costs related to the organisation of train traffic (hereinafter implementing regulation) (OJ L 148 of 13 June 2015, pages 17–22).

2.2.4 	The user fees for allowing the use of single railway capacities intended for specific purposes consist of the direct costs of railway infrastructure manager regarding the service for which mark-ups may be established based on the principles provided in section 9 of the methodology. If rail transport is performed for any purpose other than commerce, including allowing the driver of a locomotive to use the infrastructure for a practical driving test, the user fee is only related to the direct costs of allowing the capacity to be used, and mark-ups for these will not be established.

2.3 User fees for extra and support services and service facilities of the railway infrastructure

2.3.1 	The calculation of user fees for extra and support services and service facilities of the railway infrastructure is based on the total cost of services which is the sum of the direct costs related to the provided service, capital expenditure, proportional part of the overhead costs of the railway infrastructure manager and reasonable operating profit.

2.3.2 The calculation of direct costs of extra and support services only accounts for the costs related to the relevant services.

2.3.3 The overhead costs are proportionally divided between the services provided by railway infrastructure manager and the total of direct costs of all services.

2.4 Calculation of fixed assets and capital expenditure

2.4.1 Capital expenditure is the cost related to the purchase of material fixed assets.  The purpose of capital expenditure is to earn back, by selling services, the costs incurred in the acquisition of fixed assets during the service life of the fixed assets. Capital expenditure is calculated based on the depreciation rates provided in the internal rules for accounting of the railway infrastructure manager and operator of the service facility.

2.4.2 In the calculation of capital expenditure, the historical purchase value of material fixed assets, which is based on the amounts paid during the purchase of assets by the railway infrastructure manager and the operator of the service facility, is considered to be their adjusted value.

2.5 Calculation of reasonable operating profit

2.5.1 The reasonable operating profit of the railway infrastructure is calculated based on § 7 of the methodology.

2.5.2 The reasonable operating profit of the railway infrastructure manager in percentages cannot exceed the weighted average price of their capital.

2.5.3 In the calculation of reasonable operating profit, the carrying amount of fixed assets related to the provision of the relevant service as of the end of the financial year preceding the timetabling period is considered to be their adjusted value.

2.6 Unit cost of the railway infrastructure user fees

2.6.1 The unit cost of the railway infrastructure user fees is calculated on the basis of the implementing regulation.

2.6.2 The Consumer Protection and Technical Regulatory Authority may change the average direct unit costs to take into account the degree of depreciation of the railway infrastructure pursuant to the parameters set forth in article 5 point (2) of the implementing regulation.

2.7 Reduction of user fees and mark-ups

2.7.1 The Consumer Protection and Technical Regulatory Authority may only reduce the user fees of basic services ensuring access (the so-called minimum access package) to the extent of the real savings that the railway infrastructure manager has attained regarding the costs of basic services ensuring access to the infrastructure. Upon determining the rate of reduction, the decrease in costs already calculated in the current user fee cannot be taken into account.

2.7.2 Mark-ups to user fees may be reduced by the Consumer Protection and Technical Regulatory Authority pursuant to the capability of the market segment to pay user fees of railway infrastructure.

2.8 Determination of user fees

2.8.1 The Consumer Protection and Technical Regulatory Authority determines user fees for basic, extra and support services for the whole timetabling period no later than two months before the beginning of the timetabling period. User fees and mark-up rates by segments and services are published on the website of the Consumer Protection and Technical Regulatory Authority https://www.ttja.ee/et/ettevottele-organisatsioonile/raudteeinfrastruktuuri-kasutustasu.

2.8.2 Basic railway infrastructure user fee and mark-ups for user fees do not apply to work trains that move to and from the work object for the purpose of managing the railway infrastructure of Eesti Raudtee.

2.8.3 Eesti Raudtee establishes	the user fee for service facilities at least two months before the start of the timetabling period. Service facilities of the same type, e.g. storage sidings, shall have a unified user fee. Establishing the user fee for service facilities is drawn up with a decision of the Management Board. Service facility user fees shall be published by service facilities on the Eesti Raudtee website (http://www.evr.ee/et/arikliendile#eeskirjad-ja-tasud).

2.8.4 Upon establishing the user fees for service facilities, Eesti Raudtee shall adhere to the provisions of clauses 2.1.1 and 2.3 to 2.5 of the railway network statement and shall consider the causal relationships of costs with the data of the service facility users. The service facility user fee must ensure the non-discriminatory use of the service facility.

2.9 Reservation fee

2.9.1 Pursuant to § 59 (1(1)) of the Railways Act, a railway infrastructure manager may charge a reservation fee from an applicant for capacity that is allocated but not used.

3.  Principles and criteria for the allocation of capacity

## 3.1 Principles of the capacity allocation procedure

3.1.1 The purpose of the capacity allocation procedure is to satisfy applications from railway undertakings to the largest possible extent of obtaining capacity, including for such train paths that pass through more than one railway network.

3.1.2 	The railway infrastructure capacity is allocated based on the timetabling period.

3.1.3 	For using capacity, Eesti Raudtee may enter into a contract with an applicant for capacity for up to five consecutive timetabling periods, whereby the capacity is specified separately for every timetabling period. The purpose of allocating capacity is also to allocate capacity to IM/RUs that:

3.1.3.1 are capable of actually using the capacity allocated to them;

3.1.3.2 are solvent, i.e. capable of performing the obligation to pay the user fees and obligations assumed during the whole timetabling period of 2019/2020.

    - In assessing what is provided in clauses 3.1.3 of the railway network statement, Eesti Raudtee primarily takes into account the adherence to technical and safety rules by the railway undertaking, the current market situation and its opportunity for development, circumstances provided in subsection 18 (2) of the Competition Act, prior performance of the obligations arising from the contract for the use of railway infrastructure, including an evaluation on the performance of contractual agreements certified by an administrative act or court judgment that has entered into force, and other such circumstances.

    - Pursuant to subsection 50 (2) of the Railways Act, Eesti Raudtee allocates the train paths necessary for the public transport of passengers in international direct connection in the first priority. First priority applies to railway undertakings engaging in public transport of passengers in international direct connection according to international agreements who comply with the terms and conditions of such agreements.

    - Pursuant to subsection 50 (3) of the Railways Act, train paths necessary for domestic public transport of passengers are allocated in the second priority. The Ministry of Economic Affairs and Communications notifies Eesti Raudtee of the need to carry out domestic public passenger transport by the date specified in subsection 52 (1) of the Railways Act, which is 9 months before the beginning of the timetabling period.

3.1.7 	The capacity allocation procedure is conducted in a manner that the capacity to be allocated is guaranteed to be utilised to the largest possible extent. The purpose is to ensure the usage of allocated capacity to the extent of at least 80% in the timetabling period.

3.1.8	In conducting the capacity allocation procedure, Eesti Raudtee acts in the interests of all applicants, Eesti Raudtee as the railway infrastructure manager, the field of railway undertakings and society as a whole, considering the principles provided in subclauses of clause 3.1 of the railway network statement.

## Applicants for capacity and criteria for satisfying the applications

    - Applicants for capacity may be railway undertakings who hold an operating licence for rail transport of passengers or goods and a safety certificate, and, in case of applying for a single railway capacity intended for specific purposes, an applicant may also be a possessor of railway vehicles who is not a railway undertaking.

    - A railway infrastructure manager who is not a railway undertaking may apply for capacity if they are obligated to act on behalf of a railway undertaking that has applied for a train path running through more than one railway network. The capacity is applied for to be used by and at the cost of the railway undertaking.

    - Railway transport undertakings that are not established in any Member State of the European Union may be given a part of capacity by a railway infrastructure manager if the part of capacity remains unallocated for the reason that no application has been submitted for it by a railway undertaking established in a Member State of the European Union.

    - Applications for the allocation of capacity are submitted to Eesti Raudtee. The application must contain the exact business name, registry code and address of the applicant. The application must be accompanied by a document which indicates the authorisations of the person who signed the application to submit the application. The application must be prepared in Estonian and signed by the applicant. The prepared application cannot contain amendments or updates. If annexes to the application are in a foreign language, the application must be accompanied by an Estonian translation of the foreign-language annexes.

3.2.5 Upon submitting an application for the allocation of capacity, the applicant certifies that:

      - they meet the requirements provided in legal acts;

      - there are no legal or economic barriers to allocating the capacity to them;

      - they have acquainted themselves with the relevant conditions for the use of railway infrastructure provided in clause 3.9 of the railway network statement and they shall comply with them;

      - they shall use the capacity allocated to them pursuant to the principle provided in clause 3.10 of the railway network statement throughout the capacity distribution period;

      - they have acquainted themselves with the requirements originating from legal acts governing the transport of hazardous goods and they shall comply with them, if they wish to use the capacity for the transport of hazardous goods.

3.2.6	To provide proof of the circumstances indicated in clause 3.2.5 of the railway network statement, the applicant includes at least the following documents and clarifications certifying financial-economic and technical capability to perform the actual and possible obligations of the railway undertaking, at the same time determining which information contains business secrets of the railway undertaking:

      - information about contracts that certify and include the organisation of transport of goods by the applicant;

      - information about the number of passengers previously transported on railway sections and train paths in domestic public passenger transport;

      - information about the need to carry out domestic public passenger transport pursuant to the order of the Ministry of Economic Affairs and Communications;

      - information about the number of locomotives indicated in clauses 1.6.4 and 1.6.8.3.1 that are in possession of the IM/RU and are registered or are to be registered in the railway traffic register. In the absence of locomotives, the railway undertaking shall provide documents which certify the purchase/rental of a railway vehicle for the timetabling period. For a locomotive that has been entered into the railway traffic register of another state, the railway undertaking shall provide an excerpt of the relevant register;

      - the number of locomotive drivers and assistant locomotive drivers of the undertaking;

      - upon transporting hazardous goods in the 2019/2020 timetabling period, information about safety adviser and the requirements established for the transport of hazardous goods;

      - the balance sheet and income statement for 2017 as of 31 December 2017 of the undertaking, if the railway undertaking is unable to provide a certified annual report for 2017 by the time of submitting the application, and if these are unable to be accessed in public registers;

      - information about the owners and belonging to a group of companies;

      - any other information that the applicant can use to certify their ability to utilise the applied capacity to the largest possible extent.

3.2.7 Eesti Raudtee verifies the existence of the required documents provided by the applicant and the accuracy of the information provided. If the applicant has not provided all the required documents or data with the application, and they do not provide them in the additional time allocated by Eesti Raudtee to rectify the deficiencies, and also if it has been identified that the applicant has provided false information, Eesti Raudtee has the right to dismiss the application for capacity allocation in part or in full (regarding all or some parts of capacity). Eesti Raudtee notifies the applicant of a full or partial dismissal of the application without delay.

3.2.8	To verify the circumstances provided in the application, Eesti Raudtee may make queries to state databases, ask the management board and supervisory board of the applicant for clarifications about the economic status of the applicant and other circumstances that are relevant in making a decision for allocating capacity. Additional information and documents are provided in the reasonable time determined by Eesti Raudtee. If financial guarantees have been provided with the application or its clarifications, only those that have been issued by a credit institution located in a Member State of the European Union for which international credit rating agencies have issued a credit rating level Baa1 (Moody’s) and / or BBB+ (S&P and Fitch) or higher.

3.2.9 If Eesti Raudtee has identified that allocation of capacity to the undertaking is unwarranted due to nonconformity with the requirements provided in clause 3.2.6 of the statement, they shall immediately notify the applicant thereof.

## Deadlines of the capacity allocation procedure

    - The railway infrastructure capacity allocation period coincides with the timetabling period and lasts for 12 months. Applications for the allocation of capacity must be submitted to Eesti Raudtee at least 9 months before the beginning of the timetabling period.

    - Eesti Raudtee shall publish the allocation of capacity decisions to the applicants at least 7 months before the beginning of the timetabling period.

    - Eesti Raudtee prepares a draft of the timetable for the next capacity allocation period and publishes it on their website four months after the deadline provided in clause 3.3.1. Eesti Raudtee confirms the timetable at least two months before the start of the timetabling period.

## Preparation of timetable

    - The bases for preparing the timetable are:

      - requirements established in the operating rules of Eesti Raudtee;

      - coordination of capacity allocation in the Narva - Ivangorod, Pechory - Koidula and Valga - Lugaži border stations with OAO RŽD and AS Latvijas Dzelzcelš which are performed in the September of every year;

      - guidelines of the Consumer Protection and Technical Regulatory Authority for preparing the timetable;

3.4.1.4	applications submitted for obtaining capacity and, in case of a coordination process, decisions regarding the allocation of capacity;

3.4.1.5	the following data required for the preparation of a timetable submitted by railway undertakings that obtained parts of capacity pursuant to a co-ordinating decision of capacity allocation to Eesti Raudtee within two weeks of making the co-ordinating decision:

- schematics for how the locomotive fleet is situated;

- standards for the residing of locomotives in the main and turnaround-depot;

- work areas of locomotive crews and the permitted duration of working time;

- warranty areas for servicing the rolling stock;

- standard times for technical operations of processing trains in railway stations;

- local schematics for delivery of goods;

- types of locomotives and trains of railway undertakings for determining open track travelling times;

- planned standards for the distance travelled by locomotives in 24 hours;

- work technology with block trains and decoupling trains,

- transfer times of freight trains of railway undertakings in border stations,

- circulation schematics of railway vehicles for passengers,

- weights of passenger trains;

- proposals for the arrival and departure times of passenger trains by days of the week;

- stopping time of passenger trains in stations and stopping points;

- information about trains for ensuring connections in different directions for passengers.

    - After reviewing capacity applications and allocating capacity, coordinating the information obtained from railway undertakings, coordinating the timetable of international trains, Eesti Raudtee prepares a draft timetable that is available on the internet at the Eesti Raudtee website http://www.evr.ee/et/arikliendile#liiklusgraafikud.

    - In the case of a coordination process, the draft timetable will be prepared after and pursuant to the acceptance of the capacity allocation proposal provided in clause 3.5.8 of the railway network statement, or after and pursuant to the co-ordinating decision provided in clause 3.5.10. If capacity is declared depleted, the draft timetable will be prepared after and pursuant to confirming the results of auctioning the access fee provided in clause 3.7.1 of the railway network statement.

    - Interested parties have the right to present a written opinion on the draft timetable within 30 days of the publication of the draft timetable. Eesti Raudtee takes the presented opinions into account as much as possible.

3.4.5	After making the decisions for capacity allocation and receiving opinions on the draft timetable, Eesti Raudtee prepares the timetable and enforces it.

## Coordination process for the allocation of capacity

    - Should it emerge in the review of applications for capacity that more than one applicant desires the same capacity or overlapping capacity, then Eesti Raudtee is required to organize a coordination process. The coordination process of capacity allocation is the activity of Eesti Raudtee for fitting all applications in the timetable by way of negotiations with and making proposals to applicants in a manner suitable for all to the largest extent. Should the total volume of justified applications surpass the capacity to be allocated, then the capacity is declared depleted and the procedures of the commenced coordination process will not be performed.

    - Eesti Raudtee immediately notifies qualified applicants of commencing the coordination process, the capacity conflicts that caused it and announces the time and place of holding the coordination meeting. The coordination meeting is organised separately for railway passenger transport undertakings and railway freight transport undertakings. The coordination meeting is held in a reasonable time from commencing the coordination process. All qualified applicants are invited to the coordination meeting.

    - An applicant shall submit their initial opinions on the possibility and ways of resolving the capacity conflict, its effect on the economic activities of the applicant, and submits other relevant information possibly affecting the co-ordinating decision within 5 working days of receiving the notice on the commencement of the coordination process. Eesti Raudtee may operatively ask additional and specifying questions. The applicant shall submit the replies in writing at least 5 working days before the coordination meeting is held.

    - Eesti Raudtee prepares the coordination meeting by analysing applications, documents attached to these, opinions of applicants submitted under clause 3.5.3 of the railway network statement and documents attached to these. In the interest of a more effective organisation of the coordination meeting, Eesti Raudtee may make specific proposals to applicants:

      - offer a capacity different from what was applied for by volume of transport of goods and number of passengers;

      - choose a train path that passes through the railway infrastructure during a less congested time (if a time has been indicated in the application) for the provision of service;

      - choose a different route than what was designated in the application for the provision of service;

      - accelerate passing of a passenger train through a section of the railway infrastructure by reducing stops;

      - accelerate passing of a passenger train through a section of the railway infrastructure by reducing its weight or by using more powerful locomotives or multiple-unit trains;

      - increase the weight of a freight train or accelerate its passing through a section of the railway infrastructure by using more powerful locomotives.

3.5.5	An applicant shall provide a written reply to a specific proposal at the coordination meeting, at the latest.

    - At the coordination meeting, negotiations are held jointly with all applicants over the removal of conflicts over capacity in a manner that would damage the interests of all applicants the least and that would correspond to the principle of effective management of the Eesti Raudtee railway infrastructure. The course of the coordination meeting, opinions of participant, discussions and agreements are recorded. If an applicant does not partake in the coordination meeting, they are considered to have agreed to the proposal made under clause 3.5.4 of the railway network statement.

    - Eesti Raudtee shall prepare a written proposal for the allocation of capacity within five working days of holding the coordination meeting. In doing this, Eesti Raudtee shall adhere to the principles established in section 16 of the Competition Act, circumstances clarified during the coordination meeting, interests of the applicants, the need for an effective use of the railway infrastructure and the principles provided in clause 3.1 of the railway network statement. The descriptive part of the proposal includes a short overview of the reasons for organising the coordination process, applicants who submitted applications in the process thereof, the opinions and declarations of Eesti Raudtee and the course of the coordination meeting. The main part of the proposal includes the proposal and the reasoning for it. The proposal for the allocation of capacity is sent to applicants within 10 working days of holding the coordination meeting.

    - Applicants are required to give notice of their agreement or disagreement with the capacity allocation proposal within 5 working days of receiving the proposal. If an applicant does not respond by the due date, then Eesti Raudtee shall inquire by phone or e-mail whether the nonresponding applicant received the proposal. If the nonresponding applicant has received the proposal or if they cannot be reached, the applicant is considered to have agreed with the capacity allocation proposal. A written legal instrument is prepared on performing the inquiry where the person who performed the inquiry, the time, manner and result of performance are indicated, and it is kept with the coordination process documents.

    - If all applicants agree or are considered to have agreed with the capacity allocation proposal, then Eesti Raudtee shall prepare a draft timetable under the conditions provided in clause 3.4 of the railway network statement and publish it on the website http://www.evr.ee/et/arikliendile#liiklusgraafikud. The capacity allocation procedure shall continue in the manner indicated in clause 3.4.4 of the railway network statement.

    - Should one or several applicants not agree with the capacity allocation proposal, Eesti Raudtee shall publish a notice of the preparation of the co-ordinating decision on their website www.evr.ee. The co-ordinating decision is prepared by Eesti Raudtee pursuant to the principles and information provided in clause 3.5.6 of the railway network statement. The volume of capacity allocated to the railway undertaking by sections of railway infrastructure and, upon a conflict over times of passing through, also the time of passing through the section is determined with the co-ordinating decision. The co-ordinating decision shall be confirmed by the Chairman of the Management Board of Eesti Raudtee within 10 working days of publishing the notice on the preparation of the co-ordinating decision and sent in writing to all parties involved.

    - Upon disputing the co-ordinating decision, the capacity is used pursuant to the timetable prepared based on the disputed decision. In the event that a co-ordinating decision is annulled, a new capacity allocation procedure shall be organised. Until a new timetable is confirmed, the timetable that was prepared on the basis of the annulled co-ordinating decision shall be implemented.

## Declaring the capacity depleted

    - If the total volume of justified applications submitted by qualified applicants for a given railway section exceeds the capacity subject to allocation in that section as specified in Annex 2 to the railway network statement and it is therefore not possible to satisfy all applications, then Eesti Raudtee declares the capacity depleted in that section and immediately notifies the Consumer Protection and Technical Regulatory Authority and the Competition Authority thereof.

    - Upon preparing to make a decision on the depletion of capacity, Eesti Raudtee has the right to inspect the justification of applications for the allocation of capacity. Eesti Raudtee is not required to declare all railway sections with a deleted capacity to be depleted if, in the process of allocating the capacity of a railway section or sections that have been declared partially depleted, depletion lapses in other depleted sections of the railway infrastructure.

    - Capacity allocation procedure for sections that have been declared depleted shall continue as provided in clause 3.7 of the railway network statement.

## Allocation of capacity in the event that capacity has been declared depleted

    - In the event that capacity is declared depleted, Eesti Raudtee shall organise the allocation of capacity by way of auction, except for the capacity guaranteed to railway undertakings that have priority. The auction conditions and its procedure is determined by Eesti Raudtee in a manner which ensures getting the highest possible access fee together with access of railway undertakings to the railway infrastructure without discrimination, and which prevents redistribution of the market independently from market competition by way of allocating capacity.

    - Qualified applicants who applied for capacity in a railway infrastructure section that was declared depleted shall partake in the access fee auction (hereinafter bidders). The auction is organised in a written or oral format for every section of railway infrastructure the capacity of which was declared depleted.

    - The access fee auction is prepared and conducted by Eesti Raudtee. In the course of the access fee auction, Eesti Raudtee shall notify the bidders indicated in clause 3.7.2 of conducting the auction, prepares the auction, organises the registration of bidders and recording of the auction, conducts the auction, confirms the minutes and results of the auction.

    - Eesti Raudtee shall notify the bidders of organising the access fee auction via the website of Eesti Raudtee www.evr.ee at least two weeks before the auction is held. The notice includes a description of the capacity enabled to be used on the basis of the access fee, the manner of conducting the auction (written or oral), its time and place, the reasonable limit of the auction and, in case of an auction with a starting price, the starting price. The rules and regulations specifying the procedure of the auction are attached to the notice, if necessary.

    - Capacity is allocated to the highest bidder, taking into account the reasonable limit which considers the market situation. In determining the reasonable limit, the aim for allocating the capacity and the volume of capacity allocated to railway undertakings in previous timetabling periods and the practice of its real use (primarily regarding the relevant sections that have been declared depleted) are considered. Eesti Raudtee may consider other circumstances originating from the applications of qualified applicants and their annexes, such as the ability of railway undertakings to use the capacity applied for, long-term investments made by railway undertakings to provide rail transport services, and the proportion of investments made to railway undertakings conducted in Estonia. The circumstances addressed last are evaluated on the basis of applications of qualified applicants and their annexes.

    - The reasonable limit that considers the market situation is determined pursuant to the actual market segment which eliminates the allocation of capacity that exceeds or remains below the actual need for capacity. The reasonable limit is set by sections of railway infrastructure with a capacity that has been declared depleted, considering the principles provided in clause 3.7.5 of the railway network statement and the percentage of actual use of capacity of the relevant section by the railway undertaking with the largest market share. An integer of train paths is determined on the basis of this percentage; if an integer of train paths does not correspond to the percentage, the number of train paths shall be rounded to the next integer.

    - Capacity is put on the access fee auction by single train paths. The train path that corresponds to the capacity is considered to be allocated to the highest bidder for access fee, if the access fee offered by them corresponds to at least the starting price. In the absence of a bid that corresponds to at least the starting price, capacity shall not be allocated and a repeat auction shall be organised, the starting price in which may be lower than the starting price of the previous auction. Information about the capacity that remained unallocated in the auction and its allocation conditions are published on the website of Eesti Raudtee. Upon receiving an application for said capacity, Eesti Raudtee has the right to immediately satisfy it.

    - A bidder may not make a bid on capacity which exceeds the reasonable limit. A bid is dismissed in the case where it exceeds the reasonable limit. If the bidder who made the highest bid has obtained the right to use the number of train paths which corresponds to the reasonable limit, the highest bidder is removed from access fee auction for the specific section and the auction for the remaining capacity is commenced with the participation of other applicants. The starting price of the remaining capacity is 75% of the final bid of the railway undertaking who obtained the reasonable limit of volume.

    - Results of the access fee auction are confirmed by a committee within 3 working days of holding the auction. On the basis of confirmed auction results, the capacity allocation procedure shall continue in the manner indicated in clause 3.4.5 of the railway network statement.

# Application for and allocation of single partial railway capacities intended for specific purposes

    - In order to use a single railway capacity intended for specific purposes, a railway undertaking or possessor of railway vehicles shall submit an application to Eesti Raudtee at least 5 working days before the planned date of use of capacity. The application must specify the applicant’s details, the station of origin, the station of destination, the desired date and time of train movement, type and weight of railway vehicle and the purpose of transport. If the possessor of the railway vehicle is not a railway undertaking, the application must be accompanied by a confirmation from the railway undertaking for performing transport for a specific purpose in the name of the railway undertaking. Additionally, in international passenger transport, international agreements and contracts entered into between railways are followed in the application for, allocation and use of single railway capacities intended for specific purposes.

    - Eesti Raudtee reviews the submitted application and prepares the use of the single railway capacity intended for specific purposes or refusal to allocate it with the relevant reasoning within five working days as of receiving a proper application.

    - If no contract for the use of railway infrastructure governing the use of a single railway capacity intended for specific purposes has been entered into by an applicant applying for a single railway capacity intended for specific purposes or a railway undertaking performing a transport operation for specific purposes in the name of a possessor of railway vehicles pursuant to a contract provided in clause 3.8.1, then the relevant contract must be entered into before the allocation of the single railway capacity intended for specific purposes. Eesti Raudtee does not allow the use of single railway capacities intended for specific purposes prior to entering into a contract.

3.9 Contract for use of railway infrastructure

3.9.1	A railway undertaking shall timely order sufficient capacity for performing rail transport operations which they have the right to use in the volume allocated to them based on the contract for use of railway infrastructure.

3.9.2 	The contract for use of railway infrastructure is entered into with railway undertakings on equal circumstances and equal conditions for up to five consecutive timetabling periods. Different contract conditions may be applied with respect to different railway undertakings (except for the conditions established in clause 3.9.5 of the railway network statement) only in justified cases and this is not considered to be a violation of the prohibition of discrimination against railway undertakings. The special conditions may pertain to data related to the undertaking, the railway vehicles
necessary for providing a railway transport service, be dependent on the type of transport (passenger/freight transport), etc.

3.9.3 	A railway undertaking that conforms to the conditions provided in clause 3.9.4 of the railway network statement and which has been allocated capacity and issued safety certification shall enter into a contract for the use of railway infrastructure with Eesti Raudtee. The contract, the material conditions of which are listed in clause 3.9.5 of the railway network statement, is entered into in writing at least 1 month before the start of the timetabling period, whereas Eesti Raudtee shall disclose the draft contract to the railway undertaking to whom the capacity was allocated no later than 2 months before the beginning of the timetabling period.

3.9.4	Eesti Raudtee is not obligated to grant the use of the railway infrastructure to the railway undertaking and the railway undertaking does not have the right, regardless of whether the contract for the use of railway infrastructure has been entered into, to commence using the railway infrastructure before all the following conditions have been met:

3.9.4.1 the railway undertaking is in conformity with all of the requirements for railway undertakings originating from legal acts and the rules established by Eesti Raudtee, and it has all of the required permits, certificates, registrations and procedures governing the railway transport activities of railway undertakings;

3.9.4.2	the railway undertaking has, for the 2019/2020 timetabling period, valid insurance conforming to the requirements of clause 3.9.5.4 of the railway network statement and it has submitted a copy of the policy certifying it to Eesti Raudtee;

3.9.4.3 upon having an obligation to pay an access fee, they have paid the access fee to Eesti Raudtee in full;

3.9.4.4 the railway undertaking lacks any previously assumed obligations to Eesti Raudtee which arise from the contract for the use of railway infrastructure and which have become due, including the amounts payable for access to extra and support services of the railway infrastructure in the last month of the previous timetabling period, fees for basic services ensuring access to the railway infrastructure which exceeded the advance payment due to a specification of user fees, and justified contractual penalties originating from the contract;

3.9.4.5 the railway undertaking has made the advance payment established in the contract for the use of railway infrastructure or presented a bank guarantee;

3.9.4.6 undertakes to use the capacity allocated to it;

3.9.4.7   in the case of international freight transport:

3.9.4.7.1 the railway undertaking is able to plan the transport independently in accordance with the applicable procedure or order it from Eesti Raudtee under a separate contract and for a fee;

3.9.4.7.2 in the case of international freight transport, the railway undertaking has entered into a contract for use of the traffic organisation information system with Eesti Raudtee;

3.9.4.8 in the case of international freight transport, the railway undertaking is prepared to partake in the handling of complaints, including preparing legal instruments, providing explanations and testimony.

    - The railway undertaking may use the railway infrastructure pursuant to the contract for the use of railway infrastructure which establishes, among other things:

3.9.5.1 the capacity allocated to the railway undertaking for the timetabling period, conditions for applying for additional capacity, including the deadline for submitting the application for capacity three working days in advance, the calculation of use of capacity, and the deadline for the use of railway infrastructure;

3.9.5.2 the fees to be paid for using the railway infrastructure, including advance payments and conditions for the payment of these;

3.9.5.3 in the case that a contract for use of railway infrastructure is entered into for more than one consecutive timetabling period, the capacity for each timetabling period shall be specified separately;

3.9.5.4 the obligation of the railway undertaking to enter into a liability insurance contract with an insurance undertaking registered in a Member State of the European on the following conditions:

3.9.5.4.1 the amount of insurance coverage shall be 6,400,000 euros for freight transport undertakings and 1,917,350 euros for passenger transport undertakings both for a single loss event as well as the whole insurance period;

3.9.5.4.2 the liability insurance contract must be valid for the whole term of the contract for use of railway infrastructure;

3.9.5.4.3 the liability insurance contract shall cover the personal injuries and property damage suffered by both third parties as well as Eesti Raudtee;

3.9.5.4.4 the liability insurance contract shall cover compensation for direct property damage originating from activities related to transport services during the term of the contract for use of railway infrastructure for which the railway undertaking is liable pursuant to legal acts, including compensation for personal injuries and compensation for environmental damage caused by unexpected or unforeseen events. The minimum amount of insurance coverage for environmental damages shall be 10 percent of the sum insured. The liability insurance contract is not required to cover compensation for direct property damage suffered by owners of goods for which the railway undertaking is liable pursuant to contracts entered into;

3.9.5.4.5 a railway undertaking shall notify Eesti Raudtee of any amendments to the conditions of the liability insurance contract within 5 (five) working days. Conditions listed in subclauses of clause 3.9.5.4 cannot be amended by the railway undertaking;

3.9.5.5	 organisation of traffic, in addition to that which is provided in the other aforementioned rules;

3.9.5.6 conditions for the cancellation of a contract for the use of railway infrastructure;

3.9.5.7 circumstances that are considered to be material breaches of the contract for the use of railway infrastructure;

3.9.5.8 consequences of the termination of a contract for the use of railway infrastructure;

3.9.5.9 liability and procedure of dispute resolution of the railway undertaking and Eesti Raudtee.

3.9.6 If a railway undertaking with priority in the allocation of capacity and Eesti Raudtee do not reach an agreement on the conditions for the use of railway infrastructure for the timetabling period or calendar year, then Eesti Raudtee shall enable the railway undertaking to use the railway infrastructure on the conditions agreed for the previous timetabling period or calendar year, except for the user fee and mark-ups, until a new agreement is reached. If a railway undertaking receives priority for the first time and does not reach an agreement with Eesti Raudtee over the conditions for the use of railway infrastructure for passenger transport, the conditions for the use of railway infrastructure shall be determined by the Director General of the Consumer Protection and Technical Regulatory Authority, considering the conditions of contracts entered into with other railway undertakings providing public passenger transport, until an agreement is reached.

## 3.10 Obligation to use capacity

3.10.1	A railway undertaking has no right to transfer or assign railway capacity or a part thereof which has been allocated to it. In the event of a violation of the prohibition, Eesti Raudtee shall withdraw the railway capacity allocated to the railway undertaking. Applications for the allocation of capacity (except ad-hoc trains) of the railway undertaking in violation of the requirement are dismissed and not satisfied.

3.10.2	A railway undertaking to which capacity has been allocated is obligated to make the most efficient use possible of the capacity allocated to it. The efficiency of use of the railway infrastructure is taken into account in the allocation of capacity to the railway undertaking in the following timetabling periods.

3.10.3	A railway undertaking must use, in each month, at least 80% of the capacity allocated to them in railway sections that have been declared depleted.

3.10.4	A notice about the vacancy of capacity withdrawn on the basis of clauses 3.10.1 and 3.10.3 of the railway infrastructure statement is published by Eesti Raudtee on their website http://www.evr.ee/et/arikliendile#liiklusgraafikud. Eesti Raudtee determines the conditions for submitting an application for the withdrawn capacity in the notice. Withdrawn capacity is allocated pursuant to the principles established in subsection 60 (3) of the Railways Act.

## 3.11 Unallocated capacity

3.11.1 Information on unallocated capacity due to no applications, as described in clause 3.3.1, or some other reason, shall be published on the Eesti Raudtee website http://www.evr.ee/et/arikliendile#liiklusgraafikud.

3.11.2 With freight transport, the duration of 24 hours is considered to be from 17.00 to 17.00; with passenger transport, it is considered to be from 00.00 to 00.00. Eesti Raudtee shall allocate unallocated capacity by calendar months and the minimum period is one (1) calendar month.

3.11.3 Railway freight transport undertakings shall submit an application for obtaining unallocated capacity for the next calendar month to Eesti Raudtee at the e-mail address llvtellimine@evr.ee by the 25th day of the current month, indicating the total amount of capacity by railway sections required in the next calendar month in the application.

3.11.4 Eesti Raudtee shall allocate capacity based on the operational data of railway traffic. 0.5 pairs of passenger trains in 24 hours is considered the minimum use of allocated capacity in a railway section. A railway freight transport undertaking may use capacity in a calendar month pursuant to the actual need in 24 hours without exceeding the volume allocated to them for the relevant period (either a calendar month or a shorter period in the relevant calendar month). The capacity that is allocated to railway undertakings for the current calendar month that is left unused shall not be carried over to the following calendar month.

In case a railway freight transport undertaking observes the full utilisation of the total capacity allocated to them for the current month within said month, they shall submit an application for an extraordinary allocation of unallocated capacity to Eesti Raudtee by the 28th date of the current month. The application for an extraordinary allocation of unallocated capacity for freight transport must be submitted to Eesti Raudtee at the e-mail address llvtellimine@evr.ee by 10.00 of the day preceding the use of capacity, and for passenger transport, at least 11 days ahead of the date of ordered transport. Eesti Raudtee shall review the extraordinary application for freight transport within one working day and for passenger transport within four working days.

3.11.5 The railway undertaking, who has not submitted an application for the deadline established in clause 3.3.1, shall add the following documents to the capacity allocation application which corresponds to the requirements provided in clause 3.2.5 of the railway network statement:

- a copy of the policy certifying taking out a liability insurance which corresponds to the requirements provided in clause 3.9.5.4 of the railway network statement;

- a copy of the safety authorisation for transport of passengers or freight;

- a copy of the operating licence issued to perform transport of passengers or freight;

- a list of railway vehicles used for transport;

- the transport rules of the railway undertaking;

- upon a justified request by Eesti Raudtee, documents certifying the solvency of the railway undertaking.

## Plan for increasing capacity

3.12.1 Within six months of declaring the capacity of railway infrastructure depleted, Eesti Raudtee conducts an analysis of capacity. The purpose of the analysis is to determine the reasons for the depletion of capacity and financial and technical measures for eliminating the depletion and achieving additional capacity.

3.12.2 Users of railway infrastructure are consulted in conducting the analysis of capacity for the purpose of collecting information. After the analysis of capacity is conducted, Eesti Raudtee submits the results of the analysis to the railway undertaking and the Consumer Protection and Technical Regulatory Authority for submission of opinions and proposals.

3.12.3 Eesti Raudtee prepares the capacity increase plan within six months of conducting the analysis of capacity. In preparing this plan, Eesti Raudtee takes into account the proposals of users of the railway infrastructure.

3.12.4 The plan for increasing capacity is submitted to the Consumer Protection and Technical Regulatory Authority for approval. Eesti Raudtee publishes the plan for increasing railway infrastructure capacity on their website www.evr.ee after it has been approved by the Consumer Protection and Technical Regulatory Authority.

3.12.5	Railway undertakings may use sections of the Eesti Raudtee railway infrastructure that have been declared depleted without paying the access fee if:

3.12.5.1 Eesti Raudtee has not prepared a plan for increasing the capacity of the railway infrastructure by the deadline provided in clause 3.12.1 of the railway network statement; or

3.12.5.2 Eesti Raudtee does not start implementing the measures provided in the plan for increasing the capacity of the railway infrastructure within 1 (one) year of their approval by the Consumer Protection and Technical Regulatory Authority; or

3.12.5.3 Eesti Raudtee does not utilise access fees for implementing the measures provided in the plan for increasing the capacity of the railway infrastructure.

3.12.6 Railway undertakings are not relieved from the obligation to pay the access fee if implementation of the measures provided in the plan for increasing the capacity of the railway infrastructure was obstructed due to reasons irrespective of the railway infrastructure manager.

3.12.7 Should the reason for breaches indicated in clauses 3.12.5.1 to 3.12.5.3 of the railway network statement be caused by a wrongful act or non-performance by Eesti Raudtee, and Eesti Raudtee does not begin to remedy these breaches within 1 (one) year of the lapsing of the due date of performance, the railway undertaking becomes entitled to the right to claim reimbursement for the access fee together with statutory interest calculated on it. The detailed procedure of reimbursing the access fee and paying the statutory interest, including the procedure for making a claim, reimbursement deadline, the extent of calculating the interest and other circumstances shall be established in the contract for use of the railway infrastructure in the case that paying the access fee is a prerequisite for the railway undertaking to exercise their rights originating from the contract.

4.  Application for an operating licence and a safety certificate

4.1 Operating licence

4.1.1 	Pursuant to the Railways Act of the Republic of Estonia, an undertaking must have an operating licence for operation in the following areas of activity:

- management of public railway infrastructure;

- rail transport of passengers;

- rail transport of goods;

- maintenance and repair of railway vehicles used on public railways or railway vehicles used for rail transport;

- construction of railway vehicles.

4.1.2 A foreign railway undertaking need not hold an operating licence in order to enter a railway frontier station if a foreign IM/RU and an Estonian IM/RU have, on the basis of an international agreement, agreed on cross-border rail transport.

4.1.3 Applications for operating licences for management of public railway infrastructure, rail transport of passengers and rail transport of goods shall be adjudicated by the Competition Authority. Specifications for application, validity and revocation of operating licences are established in clauses 10 to 19 of the Railways Act.

4.2 Safety certificate

4.2.1 A railway undertaking may engage in transport of passengers and transport of goods on public railways if it has valid parts A and B of the safety certificate.

4.2.2 Part A of the safety certificate is issued to an undertaking that has a safety management system which complies with the requirements of the Railways Act.

4.2.3 Part B of the safety certificate is issued to an undertaking whose railway vehicles and staff comply with the requirements of the Railways Act and legislation issued on the basis thereof and if the undertaking is able to comply with the requirements for railway safety.

4.2.4   	Part A of the safety certificate issued by another Member State of the European Union to an undertaking registered in that Member State is valid in Estonia to the extent of transport specified in part A of this safety certificate for engaging either in transport of passengers or transport of goods. Pursuant to the Railways Act, the existence of part A of the safety certificate is a prerequisite for the issue of part B of the safety certificate to the undertaking of a Member State of the European Union.

4.2.5 Parts A and B of the safety certificate are issued by the Consumer Protection and Technical Regulatory Authority. The application for, and issue, amendment and renewal of safety authorisation and operational safety certificate are governed by sections 20 to 23 of the Railways Act.

4.2.6 Eesti Raudtee shall only allow the rolling stock of those railway undertakings on the public railway infrastructure in its possession who have submitted and evidenced the ownership of parts A and B of the safety certificate. This includes submitting railway sections listed in part B of the certificate, on which the specific railway undertaking can operate pursuant to the law.

5. Procedure of dispute resolution and filing of complaints regarding access to the railway infrastructure and services and the performance scheme

5.1 	If an applicant believes that they have received discriminatory or other unjust treatment in the process of capacity allocation, they have the right to file a complaint with the Competition Authority.

5.2	The Competition Authority shall make a decision on the complaint within two months of receiving all the required information. With the decision, the Competition Authority shall fail to satisfy the complaint, or issue a precept for elimination of the violation to the railway infrastructure manager, setting a deadline for it.

5.3 	If the applicant or the railway infrastructure manager does not agree with the decision of the Competition Authority, they have the right to recourse in the courts. If the decision for the allocation of capacity has been challenged, the capacity shall be used pursuant to the challenged decision until the dispute is resolved.

5.4 	In the event that decisions by Eesti Raudtee are invalidated or annulled, the railway undertaking or other railway vehicle possessor has the right to seek compensation solely for direct patrimonial damage.

5.5 The applicant for capacity and the railway infrastructure manager regulate the aims related to the performance scheme with the contract for use of railway infrastructure.

6. Access to service facilities

# 6.1 Service facilities

6.1.1 Passenger waiting areas:

Contact details of service facility operator - real estate manager, phone: 6158 517

| Station | Service description |
| --- | --- |
| Tartu station | Using the waiting area |

6.1.2 Information displays for passengers

Contact details of the service facility operator - IT department, hardware management unit, phone: 6158 795

| Name of railway stop | Number of information displays |
| --- | --- |
| Urda | 1 |
| Saue | 1 |
| Keila | 1 |
| Aruküla | 2 |
| Raasiku | 2 |
| Aegviidu | 2 |
| Tapa | 2 |
| Jõgeva | 1 |
| Tabivere | 1 |
| Tartu | 1 |
| Elva | 1 |
| Jõhvi | 1 |
| Narva station, passenger waiting area | 1 |
| Tallinn-Balti passenger station, track 1 | 1 |
| Tallinn-Balti passenger station, track 4 | 2 |
| Tallinn-Balti passenger station, track 5 | 1 |
| Tallinn-Balti passenger station, track 6 | 1 |
| Tallinn-Balti passenger station, track 7 | 1 |
| Tallinn-Balti passenger station, track 8 | 1 |
| Tallinn-Balti passenger station, track 9 | 1 |
| Tallinn-Balti passenger station, on the wall of the diesel train area building | 2 |
| Tallinn-Balti passenger station, on the wall of the electric train area building | 2 |

6.1.3 Railway repair workshops:

Contact details of service facility operator - real estate manager, phone: 6158 517

| Railway repair workshop | Service description |
| --- | --- |
| Koidula | Use of railway repair workshop |

6.1.4 Station tracks (storage sidings) dedicated to the temporary parking of a railway vehicle:

Contact details of the service facility operator – station works department, head specialist of transport plans and conditions, phone: 6158831

| Station track | Usable length in km |
| --- | --- |
| Aegviidu 5 | 0.874 |
| Aegviidu 3 | 0.932 |
| Riisipere 1 | 0.785 |
| Keila 7 | 0.142 |
| Muuga 10 | 0.146 |
| Muuga 31 | 0.257 |
| Muuga 32 | 0.208 |
| Muuga 33 | 0.88 |
| Muuga 34 | 0.151 |
| Muuga 48 | 0139 |
| Maardu 11 | 1.051 |
| Narva 23 | 0.201 |
| Narva 21 | 0.266 |
| Narva 11 | 0.515 |
| Narva 22 | 0.198 |
| Tallinn-Balti passenger station, 9A | 0.353 |
| Tallinn-Balti passenger station, 11 | 0.270 |
| Tallinn-Balti passenger station, 12 | 0.254 |
| Tartu 47 | 0.144 |
| Tartu 48 | 0.191 |
| Tartu 4B | 0.432 |
| Tartu 6 | 0.880 |
| Tartu 12 | 0.676 |
| Rakvere 6 | 0.400 |
| Rakvere 1 | 0.903 |
| Tapa 14 | 0.911 |
| Tapa 21 | 0.853 |
| Ülemiste 7 | 0.100 |
| Ülemiste 11 | 0.902 |
| Ülemiste 25 | 0.742 |
| Paldiski 1 | 0.351 |
| Valga 5 | 0.891 |
| Valga 12 | 0.164 |
| Valga 13 | 0.256 |
| Koidula 10 | 0.529 |
| Koidula 100 | 0.312 |
| Koidula 101 | 0.207 |

6.1.5 Station tracks (sidings) that are usable without applying for capacity

for manoeuvring and connections with other railways:

Contact details of the service facility operator - station works department, head specialist, phone: 6158 653

| Station list | Connection location |
| --- | --- |
| Tallinn-Kopli | With connecting track No. 50, 401, 403, 404, 406, 314-315, 319-320, 1 (Paljassaare), 2 (Paljassaare) |
| Ülemiste | With connecting tracks No. 5, 200, 113, 114, 120 BAAS, 301, 403, 401, 402, track No. 3 (Sõjamäe park), track No. I |
| Muuga | With connecting tracks No. 401, 502, 503, 613, 701, 41A, 1, 3, 20/300, 22, 23, 27, 28, 32, 33 |
| Maardu | With connecting tracks No. 101, 201, 20, 501, 402, 305, 401, 300 |
| Lagedi | With connecting track No. 8 |
| Kehra | With connecting track No. 101 |
| Tapa | With connecting tracks No. 25, 61A, 106, 114, 72, 501, 700, 701, 736, 741, 742, 744, 774 |
| Rakvere | With connecting tracks No. 9A, 10, 12, 13, 15, 23, 24, 25 |
| Sonda | With connecting track No. 101 |
| Kiviõli | With connecting track No. 106 |
| Püssi | With connecting tracks No. 201, 301 |
| Kohtla | With connecting tracks No. 201, 19 |
| Jõhvi | With connecting tracks No. 101, 201, 401 |
| Vaivara | With connecting tracks No. 101, 201 |
| Soldina | With connecting tracks No. 202, 301 |
| Narva | With connecting tracks No. 38, 201 |
| Tamsalu | With connecting tracks No. 10, 16 |
| Rakke | With connecting track No. 101 |
| Jõgeva | With connecting tracks No. 7, 8 |
| Tabivere | With connecting track No. 4 |
| Kärkna | With connecting tracks No. 1A, 8, 9 |
| Tartu | With connecting tracks No. 16, 82, 39, 76, 24, 27, 28, 44, 29, 30, 43 |
| Reola | With connecting tracks No. 201, 211 |
| Põlva | With connecting track No. 101 |
| Elva | With connecting track No. 8 |
| Sangaste | With connecting track No. 10 |
| Valga | With connecting track No. 6/2, 11, 15, 16, 21, 25, 80, 82, 83, 84, 101 |
| Võru | With connecting tracks No. 10, 12, 13, 15, 1B |
| Riisipere | With connecting tracks No. 5/7 |
| Vasalemma | With connecting track No. 100 |
| Paldiski | With connecting tracks No. 101, 12, 501, 300, 301, 302 |
| Keila | With connecting tracks No. 14, 211 |
| Pääsküla | With connecting tracks No. 50/51 |

6.1.6 Turntable:

Contact details of service facility operator - real estate manager, phone: 6158517

| Station | Service description |
| --- | --- |
| Koidula | Turning around the railway vehicle |

6.1.7 Refuelling facilities:

Contact details of service facility operator - real estate manager, phone: 6158 517

| Station | Service description |
| --- | --- |
| Koidula | Refuelling the railway vehicle |

6.2  Operator of service facilities

6.2.1 The operator of service facilities is Eesti Raudtee pursuant to subsection 581 (5) of the Railways Act.

6.3 Access to service facilities and the services provided therein

6.3.1 	The operator of service facilities guarantees access to service facilities indicated in section 6.1 and the services provided therein to all railway undertakings without discrimination.

6.3.2  	The operator of a service facility shall respond to an application for access to the service facility and the services provided therein by a railway undertaking within 30 days of receiving the application.

6.3.3 	The application indicated in clause 6.3.2 may only be rejected in the case that there is another feasible alternative for providing relevant transport services in the same or alternative route on conditions that are economically reasonable. Submission of an application for access does not obligate the operator of a service facility to make investments in order to fulfil all the applications of railway undertakings.

6.3.4 	The operator of a service facility shall justify their decision to reject an application for access in writing and indicate which other feasible alternatives exist in other facilities.

6.3.5 	Should applications of railway undertakings overlap, the operator of the service facility shall try to satisfy all applications to the largest possible extent.

6.3.6 	If a service facility indicated in section 6.1 has not been used for two consecutive years and railway undertakings have expressed a wish to gain access to the facility and certified their need for access, the operator of the service facility shall immediately notify all interested parties that the facility can be operated in part or in full as a service facility under a contract for use, or due to current reorganisation operations.

7. Allocation procedure of international train paths

# 7.1 Passenger train paths

7.1.1 	Eesti Raudtee cooperates with the capacity allocation bodies of other states to organise international train paths.

7.1.2	Processing of applications for capacity required for determining the operation of international passenger trains and amendments to capacity related to international passenger trains is performed pursuant to subsection 50 (2) of the Railways Act.

7.1.3   	Determining the operation of international passenger trains occurs pursuant to the procedure in the Agreement on International Passenger Transport approved by the Council for Railway Transport (operating rules of Eesti Raudtee, clause 63 of the Annex). Railway administrations and passenger transport undertakings shall cooperate on the coordination of train paths at least 10 months before the beginning of the timetabling period. Eesti Raudtee submits proposals on the organisation of routes in international connection to relevant railway administrations at least 9 months before the beginning of the timetabling period.

7.2 Freight train paths

7.2.1	To enter freight trains into the timetable, a railway freight transport undertaking shall:

- submit the planned volume of freight transport together with the application for capacity by 01 March 2019;

- submit proposals for the international trip plan of trains in coordination with the planned volume of transport of goods and the application for capacity by 01 April 2019.

7.2.2 The timetable of international freight trains is coordinated with the railway administrations of other states during October.

8. Market segments

- Domestic passenger transport

- International passenger transport

- Unladen journeys of locomotives

- Periodic container and container trains arriving from or heading to third countries

- Domestic/Intra-European transport of waste rock:

- Domestic/Intra-European transport of timber and metal

- Domestic/Intra-European periodic container and contrailer train transport service

- Domestic/Intra-European transport service

- Import and export

- Transit via Valga railway border station (petroleum products, chemical products and grain)

- Transit via Narva railway border station (petroleum products, chemical products, except fertilisers)

- Transit via Koidula railway border station (petroleum products, chemical products and palm oil)

- Transit

# 9. Annexes to the railway network statement

The following annexes to the railway network statement are integral parts of the railway network:

Annex 1:	Operations enabled on the railway civil engineering works of Eesti Raudtee;

Annex 2:	Characteristics of railway sections and general description of capacity to be allocated;

Annex 3:	Technical data of the railway infrastructure and deadlines of planned rail maintenance works;

Annex 4:   Data for the calculation of ordered train-kilometres;

Annex 5:   Application form for unallocated capacity;

Annex 6:   Providers of a border service in frontier stations and providers of a locomotive service between Koidula and Pechory frontier stations.